PROJECT TITAN

Design Decision Log

Educational Engineering Project

| Document Number | TITAN-DDL-001 |
| --- | --- |
| Revision | A |
| Status | Released - Educational Baseline |
| Author | Michael Lozada-Longog |
| Affiliation | Biomedical Engineering, Hawaii Pacific University |
| Date | July 2026 |
| Classification | Educational Engineering Project |

IMPORTANT SCOPE NOTICE

Project TITAN is an educational design and analysis project. It is not a clinically validated device, regulatory submission, or product cleared for human use.

# Document Control and Revision History

| Revision | Date | Description | Prepared By | Status |
| --- | --- | --- | --- | --- |
| A | July 2026 | Initial retrospective decision log documenting the baseline Project TITAN design and analysis choices. | Michael Lozada-Longog | Released |

Source baseline: Project TITAN Engineering Design Report (July 2026), CAD assembly, completed engineering drawings, and Fusion 360 static FEA results documented by the author.

# 1. Purpose and Use

This log records why key engineering choices were made, which alternatives were considered, and what evidence supports each choice. Entries are retrospective where the original decision was made during CAD development before the formal documentation suite existed. Future entries should be created at the time of decision.

# 2. Decision Status Definitions

| Status | Meaning |
| --- | --- |
| Released | Accepted for the current educational baseline |
| Conditional | Accepted subject to future verification or test |
| Superseded | Replaced by a later decision |
| Open | Decision required before the next build stage |

# 3. Decision Register

| Decision ID | Decision | Alternatives Considered | Rationale | Status | Evidence |
| --- | --- | --- | --- | --- | --- |
| DDL-001 | Use a dual-side-plate architecture | Single-side hinge; four-bar linkage; powered joint | Dual plates provide a balanced, simple, and analyzable structure for the first mechanical baseline. | Released | PRD/SYS |
| DDL-002 | Use a 10 mm nominal main-shaft diameter | Smaller pin; larger shaft | The 10 mm diameter aligned with the selected simplified bearing bore and provided a conservative starting geometry. Hand calculations remain required. | Conditional | CAD/CAL future |
| DDL-003 | Use 22 mm OD × 10 mm ID × 7 mm wide simplified bearings | Bushings; custom bearing; larger bearing | Standardized geometry simplified sourcing, CAD, and the load-transfer concept. Final COTS part number is not yet selected. | Conditional | CAD/MFG future |
| DDL-004 | Use 6 mm plate and frame thickness | 5 mm; 8 mm; ribbed section | The 6 mm baseline balanced modeling simplicity and stiffness. Static FEA met the SF requirement; fatigue and physical tests remain open. | Released | FEA |
| DDL-005 | Use 60 mm-wide side plates | Narrower tapered plate; wider plate | A 60 mm baseline provided room for mounting holes, bearing bore, and lightening slots while retaining edge distance. | Released | CAD |
| DDL-006 | Reduce lightening-slot width after packaging review | Original 18 mm slot; no slots | The original 18 mm concept conflicted with the desired wall and hole edge distances. Narrower slots preserved material around load paths. | Released | CAD review |
| DDL-007 | Place two strap slots at the top of the upper frame and bottom of the lower frame | Single central slot; no slots | Separated slots provide a clear conceptual attachment feature and distinguish upper/lower frame functions. Actual strap design is outside baseline scope. | Conditional | CAD/HF future |
| DDL-008 | Use separate upper and lower frame components | Single continuous rail | Separate parts support modularity, independent drawing control, and future adjustability. | Released | PRD/CAD |
| DDL-009 | Use two spacers | No spacers; integrated shoulders | Separate spacers simplified axial-stack control and allowed individual manufacturing documentation. A tolerance stack-up remains required. | Released | CAD |
| DDL-010 | Use 6061-T6 aluminum for structural baseline | 7075-T6; 316L; titanium; composite; PEEK | 6061-T6 offered the best balance of cost, machinability, availability, corrosion resistance, and adequate static performance. | Released | MAT-001 |
| DDL-011 | Model bearings as simplified solid components | Full rolling-element geometry; omit bearings | Simplified solids reduced model complexity while preserving envelope and load-transfer surfaces for educational FEA. | Released | CAD/FEA |
| DDL-012 | Use component-level FEA before full-assembly FEA | Full assembly only | The isolated plate study validated material, constraints, load application, meshing, and solver workflow before contact-heavy assembly analysis. | Released | FEA |
| DDL-013 | Apply full-assembly loads to inner bearing surfaces | Load side-plate bore directly | Inner-surface bearing loads better represented the intended external load entering through the bearing/shaft interface. | Released | FEA |
| DDL-014 | Use a minimum safety-factor requirement of 2.0 | 1.5; 3.0 | A target of 2.0 provided a clear educational static requirement. It is not a fatigue or clinical safety factor. | Released | PRD/FEA |
| DDL-015 | Use refined mesh for final baseline results | Default mesh only | Refinement increased confidence that the displayed stress and displacement were not solely coarse-mesh artifacts. | Released | FEA |
| DDL-016 | Accept refined-study results as the current static baseline | Earlier 100 N or preliminary assembly result | The refined study used corrected bearing load placement and current constraints. Reported values: 54.95 MPa, SF 2.377, 0.004 in displacement. | Released | FEA report |
| DDL-017 | Create individual part drawings | Assembly drawing only | Individual drawings demonstrate manufacturability, part control, dimensions, materials, tolerances, and revision identity. | Released | Drawing package |
| DDL-018 | Assign sequential TITAN part numbers | File names only | Part numbers improve configuration control, BOM clarity, and drawing traceability. | Released | Drawing package |
| DDL-019 | Treat bearings as COTS concept components | Custom-manufacture bearings | Commercial bearings reduce manufacturing scope and are appropriate for the intended architecture. Final supplier selection remains open. | Conditional | MFG future |
| DDL-020 | Keep the project explicitly educational and non-clinical | Present as a finished medical device | The design lacks prototype, fatigue, human-factors, biocompatibility, and clinical evidence. Clear scope prevents unsupported claims. | Released | All documents |
| DDL-021 | Defer GD&T until it can be applied correctly | Add decorative GD&T callouts | Incorrect GD&T would reduce credibility. Datum schemes and functional tolerances will be added after fit and inspection requirements are defined. | Released | MFG future |
| DDL-022 | Defer powered actuation and sensors to V3 | Add motor/electronics to baseline | A mechanical-first baseline isolates structural learning and avoids expanding scope before the hinge is physically validated. | Released | Roadmap |
| DDL-023 | Revisit shaft material before physical V2 | Retain aluminum without review | Bearing wear and shaft surface hardness may make stainless or hardened steel preferable for a functional prototype. | Open | MAT/RSK |
| DDL-024 | Require physical validation before wearable testing | Proceed directly to human testing | FEA alone cannot validate contacts, fatigue, wear, assembly error, or misuse. Benchtop testing is required first. | Released | RSK/PRO future |

# 4. Key Decisions Requiring Revisit Before Prototype V2

| Decision | Reason for Revisit | Required Evidence |
| --- | --- | --- |
| Main shaft material and finish | Current aluminum baseline may be weak in wear and surface hardness | Bending calculation, bearing-interface review, material trade |
| Bearing retention and fits | Current assembly does not release a production retention method | Tolerance/fit study, supplier data, prototype pull-out test |
| Fastener selection | Current rigid joints are CAD abstractions | Fastener grade, size, torque, thread engagement, locking method |
| Side-plate geometry | Static pass does not establish fatigue or weight optimum | Fatigue analysis, topology study, physical strain/deflection data |
| Mechanical range of motion and hard stop | Current baseline lacks validated ROM limit | Kinematic requirement, collision review, hard-stop design and test |

# 5. Design Decision Template for Future Entries

Required Entry Fields Decision ID; date; problem statement; alternatives; criteria; selected option; rationale; supporting evidence; affected documents/parts; risks; verification action; approval/status.

# 6. Decision Quality Review

The baseline decisions show a coherent mechanical-first development strategy. The strongest decisions are those directly supported by completed evidence: modular CAD, component drawings, FEA workflow, refined static results, and material baseline. The weakest or most conditional decisions concern production fits, shaft material, fasteners, dynamic loading, fatigue, and human use because those areas do not yet have calculations or physical test evidence.

# 7. Conclusion

The design decision log converts the project history into a traceable engineering narrative. It demonstrates that Project TITAN evolved through packaging constraints, assembly troubleshooting, simulation setup correction, and documentation improvements. Future revisions should record decisions prospectively and link each entry to drawings, calculations, risks, tests, and change requests.

# References

Project TITAN Engineering Design Report, July 2026.

TITAN-PRD-001, Product Requirements Document, Revision B.

TITAN-SYS-001, System Architecture Document, Revision A.

TITAN-MAT-001, Material Trade Study, Revision A.

TITAN-RSK-001, Educational Risk Analysis, Revision A.