PROJECT TITAN

Manufacturing and Assembly Plan

| Document Number | TITAN-MFG-001 |
| --- | --- |
| Revision | A |
| Author | Michael Lozada-Longog |
| Date | July 2026 |
| Status | Released - Educational Engineering Project |

EDUCATIONAL USE NOTICE

Project TITAN is an educational engineering design project. It is not a clinical device, regulatory submission, or product approved for human use.

# Document Control

| Revision | Date | Description | Author |
| --- | --- | --- | --- |
| A | July 2026 | Initial manufacturing and assembly planning release. | Michael Lozada-Longog |

# 1. Manufacturing Strategy

The baseline metal concept uses CNC-milled 6 mm plate/frame components, turned shaft/spacers, and purchased bearings/hardware. This is a process plan, not a released production router.

# 2. Component Process Plan

| Component | Material intent | Primary process | Key operations | Inspection |
| --- | --- | --- | --- | --- |
| Main shaft | To be confirmed; drawing currently indicates 6061-T6 | CNC turning | Face, turn OD, deburr | Micrometer: diameter/length; straightness check |
| Left/right side plates | 6061-T6, 6 mm | CNC milling | Profile, slots, bearing bores, mounting holes, edge finish | Caliper, pin/bore gauge, flatness check |
| Upper/lower frames | 6061-T6, 6 mm | CNC milling | Profile, slots, holes, deburr | Caliper, hole-position inspection |
| Left/right spacers | 6061-T6 | CNC turning | Face, bore/turn, deburr | Micrometer/caliper |
| Bearings | Commercial item | Purchase | Incoming inspection only | Part number, dimensions, smooth rotation |
| Hardware | Commercial item | Purchase | Incoming inspection only | Grade, thread, length, quantity |

# 3. Recommended Operation Sequence

Review released drawing and raw-stock allowance.

Rough machine external profiles and datums.

Machine critical bores/slots after stable fixturing is established.

Deburr and break sharp edges without changing critical dimensions.

Inspect critical features before surface finishing.

Anodize aluminum only after fit validation, if selected.

Perform final inspection and serialize prototype parts.

# 4. Assembly Sequence

Clean and inspect all parts.

Install bearings into side plates using the final defined fit/retention method.

Insert shaft and spacers; verify smooth rotation and axial alignment.

Attach upper and lower frame members using specified hardware.

Install mechanical hard stop.

Torque fasteners to the future released specification and apply witness marks.

Perform final interference, motion, and dimensional inspection.

# 5. Quality Hold Points

| Hold Point | Acceptance |
| --- | --- |
| HP-1: Raw material | Correct alloy/temper and thickness documented |
| HP-2: Bearing bores | Diameter, roundness, and alignment meet released drawing |
| HP-3: Shaft | Diameter and straightness support intended bearing fit |
| HP-4: Dry assembly | No interference; smooth bearing-supported movement |
| HP-5: Final assembly | Fasteners complete; no damage; documented inspection |

# 6. Open Manufacturing Definitions

Bearing catalog part number and fit class

Shaft material and surface finish

Fastener material/grade and torque

GD&T datum scheme and positional tolerances

Anodize type and masking requirements