PROJECT TITAN

Prototype Development Plan

| Document Number | TITAN-PRO-001 |
| --- | --- |
| Revision | A |
| Author | Michael Lozada-Longog |
| Date | July 2026 |
| Status | Released - Educational Engineering Project |

EDUCATIONAL USE NOTICE

Project TITAN is an educational engineering design project. It is not a clinical device, regulatory submission, or product approved for human use.

# Document Control

| Revision | Date | Description | Author |
| --- | --- | --- | --- |
| A | July 2026 | Initial phased prototype-development plan. | Michael Lozada-Longog |

# 1. Objective

Define a low-risk progression from digital design to a measurable physical prototype while preventing unsupported wearable or clinical claims.

# 2. Prototype Stages

| Stage | Purpose | Build | Exit Criteria |
| --- | --- | --- | --- |
| P0 - Digital baseline | Confirm CAD, documentation, and static analysis | Fusion assembly and controlled document set | V1 documentation released; open issues identified |
| P1 - Fit-check prototype | Validate geometry, clearances, and assembly sequence | FDM printed structural parts plus low-cost shaft/bearing substitutes | Assembles without interference; motion observed; change list captured |
| P2 - Bench mechanical prototype | Validate real bearings, shaft alignment, and structural assembly | Machined aluminum or hybrid metal/polymer parts; commercial hardware | Dimensional inspection passed; smooth movement; safe bench load test completed |
| P3 - Instrumented bench concept | Demonstrate sensing only, not clinical function | IMU/encoder/load sensor and data logger | Repeatable angle/load data on a fixture |
| P4 - Future biomedical research concept | Explore controlled human-factors questions only after approvals | Redesigned system with safety controls | Separate review, risk assessment, and institutional approval required |

# 3. P1 Build Plan

Export current components as STL with consistent orientation.

Print plates/frames using a strong, dimensionally stable material and adequate wall count.

Use purchased bearings only if the printed bore fit is controlled; otherwise use printed bushings for fit checks.

Record every interference, gap, and difficult assembly step.

Update CAD and drawings before proceeding to metal.

# 4. P2 Bench Test Plan

| Test | Method | Preliminary acceptance |
| --- | --- | --- |
| Dimensional inspection | Caliper/micrometer/bore gauges | Critical dimensions within released tolerances |
| Rotation/kinematics | Manual movement through intended range | No binding, scraping, or uncontrolled looseness |
| Static deflection | Fixture assembly; apply incremental load below test limit | No permanent deformation; measured deflection recorded |
| Fastener retention | Torque and witness-mark review | No loosening during bench cycling |
| Post-test inspection | Visual and dimensional comparison | No cracks, yielding, or bearing migration |

# 5. Safety Boundaries

No human wear or clinical evaluation in P0-P2.

Use guards and remote loading during bench tests.

Define proof-load limits before testing.

Stop testing upon visible damage, unusual noise, or unexpected displacement.

Document deviations and failed tests; do not hide them.

# 6. Required Future Artifacts

Prototype BOM, build traveler, inspection sheet, test protocol, raw data, photographs, deviation log, and test report.