PROJECT TITAN

Requirements Traceability Matrix

| Document Number | TITAN-RTM-001 |
| --- | --- |
| Revision | A |
| Author | Michael Lozada-Longog |
| Date | July 2026 |
| Status | Released - Educational Engineering Project |

EDUCATIONAL USE NOTICE

Project TITAN is an educational engineering design project. It is not a clinical device, regulatory submission, or product approved for human use.

# Document Control

| Revision | Date | Description | Author |
| --- | --- | --- | --- |
| A | July 2026 | Initial traceability release linking requirements to current evidence and open verification work. | Michael Lozada-Longog |

# 1. Traceability Matrix

| Req ID | Requirement | Method | Evidence | Status |
| --- | --- | --- | --- | --- |
| FR-001 | Two-bearing shaft support | CAD inspection | Assembly model; 31 mm bearing-center spacing evidence | Verified |
| FR-002 | Interference-free modular assembly | CAD review | Final assembled model and exploded view | Verified |
| FR-003 | Replaceable components | Architecture review | Separate component instances and drawings | Verified by design |
| PR-001 | Minimum SF >= 2.0 | Static FEA | Reported minimum SF 2.377 | Verified for current study |
| PR-002 | Stress below configured yield | Static FEA + material audit | 54.947 MPa max; Fusion effective yield implied by SF is ~130.6 MPa | Conditional - audit required |
| PR-003 | Displacement <= 0.005 in | Static FEA | 0.004 in maximum | Verified |
| MR-001 | CNC-compatible parts | Drawing/process review | Milling and turning plan | Verified by review |
| MR-002 | Commercial bearings/hardware | BOM review | Bearing geometry present; exact catalog selections open | Partially verified |
| DR-001 | Part drawings | Document review | Completed individual drawing files | Verified |
| DR-002 | Engineering report | Document review | Project TITAN Engineering Design Report.pdf | Verified |
| VR-001 | Record loads and geometry | Evidence review | 2 x 2000 N screenshots; 6 mm thickness confirmation | Verified |
| VR-002 | Analytical sanity checks | Calculation review | CAL-001 | Verified with limitations |
| VR-003 | Physical validation | Prototype test | Future test report | Open / future |

# 2. Closure Summary

The current V1 package closes CAD, documentation, and static-FEA requirements. Material-model confirmation and physical validation remain open. Those open items are disclosed rather than treated as completed evidence.